D	TG4 File Format

TG4 XE "TG4"§(Tiled CCITT Group 4) is IMageCenter's native format. In this format, each image is segmented and stored as a collection of 512 by 512 pixel tilesXE "tiles"§, each of which is compressed in CCITT Group 4 as though it were its own free-standing image. The rare tiles that do not compress (i.e., they "negatively" compress) are flagged and maintained in uncompressed format to avoid the additional storage requirements imposed by the negative compressionXE "negative compression"§.

Tiles are numbered within the image in row-major order, i.e., tile 0 is at the image's top left corner, tile 1 is to its right, and so forth. The number of tiles in an image is always computable directly from the image's line height and bytewidth: If H is the line height of an image and W is its byte width, then the following equations always describe the image's tile geometry:

tileheight = ( lineheight + 511 ) / 512;

tilewidth = ( bytewidth + 63 ) / 64;

tilecount = tileheight * tilewidth.

Unused parts of rightmost and bottommost edge tiles are ignored, and are normally padded with 0 bytes to achieve best possible compression. Image widths must always be a multiple of 8 pixels.

A 128-byte XE "file header"§file headerXE "header"§, a tile indexXE "tile index"§, and the compresed tiles themselves fully describe a TG4 image. The 128-byte header is always at offset 0 in a TG4 image file.

•	Note: TG4 is directly compatible with the Navy EDMICS XE "EDMICS"§format, since first-generation IMC products formed the basis of that system in 1988-1989. TG4 tiles are also closely compatible with the Aldus TIFF 6.0 tiled format specifications. When IMageCenter translates to or from EDMICS, or to tiled TIFFXE "TIFF"§, only file header and index changes need to be made - a very fast operation. The data itself does not generally have to be converted.

TG4 File Byte and Bit OrderingXE "Bit Ordering"§

All multi-byte fields in the TG4 header and index follow the Intel byte-ordering (low-order  byte first). Compressed TG4 tile data is stored byte-by-byte, using a left to right bit ordering in each byte (i.e., the first bit of a tile's first scan line's compressed data is stored in the 0x80 bit of the first byte of the compressed tile data).

TG4 File Header

The following 'C' structure defines a TG4 header. Only the fields in bold, italic type are currently supported by IMageCenter. All other fields are set to 0 anytime IMageCenter creates a new header, or are ignored when reading an existing header.

| Field | Type | Byte Offset | Description |
| --- | --- | --- | --- |
| indexoffset | DWORD | 0.00 | Offset in file of index start (normally 128L, i.e. immediately following the header). |
| lineheight | WORD | 4.00 | Image line height |
| bytewidth | WORD | 6.00 | Image byte width |
| year | WORD | 8.00 | Creation date, year (i.e. 1993) |
| seconds | DWORD | 10.00 | Creation date, seconds from 12 AM, January 1 of year |
| site_id | DWORD | 14.00 | Vendor-specific site ID |
| source_id | WORD | 18.00 | Vendor-specific scanner ID |
| previewoffset | DWORD | 20.00 | File offset of image preview's header start. |
| xorigin | WORD | 24.00 | logical horizontal offset of image data. If non-zero, there are xorigin implied white pixels to the left of each scan line in the image. |
| yorigin | WORD | 26.00 | logical vertical offset of image data. If non-zero, there are yorigin implied white scan lines preceeding image. |
| displayunits | char | 28.00 | (not currently defined) |
| displaycolor | char | 29.00 | (not currently defined) |
| righttrim | char | 30.00 | unused rightmost pixels in final byte of each scan line; permits exact image pixel width specification in connection with the bytewidth field; legal values are 0-7 |
| unused1 | char [5] | 31.00 | (not currently defined) |
| format | char | 36.00 | Image data format. This value is 4 for TG4. |
| unused2 | char | 37.00 | (not currently defined) |
| dpi | int | 38.00 | Image resolution in dots per inch. |
| unused3 | char | 40.00 | (not currently defined) |
| rotation | char | 41.00 | (not currently defined) |
| unused4 | char [2] | 42.00 | (not currently defined) |
| nextheaderoffset | DWORD | 44.00 | File offset of next embedded image, if any. |
| hollerithXE "hollerith"§ | char [80] | 48.00 | Any descriptive text associated with the image (zero-terminated if fewer than 80 characters are present). |

•	Note: IMCLIB.H contains this and other structure definitions for use by system developers. IMC's SDK contains substantially more documentation on these and other aspects of the system and is available as a separate product from IMC.

TG4 Tile IndexXE "Tile Index"§

The index's offset is specified in the image header, and is normally 128L (i.e., the index immediately follows the header).  When the index offset is not 128L, it is assumed that the tile data immediately follows the header, and that the index immediately follows the tile data. (These are the only two acceptable cases.)

There is exactly one four-byte index entry for each tile. The following C structure defines each entry:

typedef struct {

WORD	tilenumber;

WORD	tilesize;

} IMAGEFILEINDEX;

tilenumber	Number of the tile within the image (low order 15 bits), and NEGFLAG (bit 0x8000).

Tile 0 is at the image's upper left corner, tile 1 is to the right of tile 0, and so forth. Note that tiles need not be stored in order within a TG4 file, although IMC products will always produce files whose tiles are in ascending order. When the NEGFLAG is set, the tile is uncompressed and must have exactly 0x8000 bytes (32K).

tilesize	Byte size of the compressed tile, or 0x8000 if the tile is marked as negatively compressed (i.e. is uncompressed).

IMageCenter always creates compressed tiles whose byte size is even (i.e. a word multiple), although this is not required by TG4. The minimum acceptable tile size is 67, since this is the compressed size of an all-white tile.

TG4 Tile DataXE "Tile Data"§

Tile data always begins either immediately following the index when the index is located at file offset 128L , or at 128L when the index is not at this ofset (i.e., when the index follows the data). In either case, tile data is stored continuously in the order described by the tile index. (The absolute file offset of any given tile can be determined via calculations on the tile index.)

TG4 Embedded PreviewXE "Embedded Preview"§

When a TG4 file contains an embedded image previewXE "image preview"§, the preview's offset within the file is specified by the header's previewoffset field. When there is no embedded preview, the previewoffset field is 0L. When present, the preview always immediately follows the main image within the TG4 file.

A preview XE "preview"§is just a scaled version of the main image, and is actually just another TG4 image that happens to be embedded in the main file at a given offset. The preview's 128-byte header begins at previewoffset, and is followed immediately by the preview's tile index, then the preview's tile data. (The index always precedes the tile data for previews.)

XE "TG4 File Example"§TG4 File Example

The contents of a typical TG4 image file are shown below. For this example, we use an all-white 200 dpi landscape E-size image (6800 lines high by 8800 pixels wide) with no Hollerith data, and attached 850 by 1104 pixel preview (1/8th scale in each dimension). The main image is 18 tiles wide by 14 tiles high (252 tiles total). Because we are assuming it is completely white, each tile's compressed size will be 68 bytes (actually 67, but IMC products round up to the nearest word boundary). The preview will be 3 tiles wide by two tiles high (6 tiles total), again with all-white tiles.

| Field | Type | Byte Offset | Value (Intel order) |
| --- | --- | --- | --- |
| indexoffset | DWORD | 0.00 | 128.00 |
| lineheight | WORD | 4.00 | 6800.00 |
| bytewidth | WORD | 6.00 | 1100.00 |
| year | WORD | 8.00 | not used |
| seconds | DWORD | 10.00 |  |
| site_id | DWORD | 14.00 |  |
| source_id | WORD | 18.00 |  |
| previewoffset | DWORD | 20.00 | 18272.00 |
| xorigin | WORD | 24.00 | not used |
| yorigin | WORD | 26.00 |  |
| displayunits | char | 28.00 |  |
| displaycolor | char | 29.00 |  |
| righttrim | char | 30.00 | 0.00 |
| unused1 | char [5] | 31.00 | not used |
| format | char | 36.00 | 4.00 |
| unused2 | char | 37.00 | 0.00 |
| dpi | int | 38.00 | 200.00 |
| unused3 | char | 40.00 | 0.00 |
| rotation | char | 41.00 | not used |
| unused4 | char [2] | 42.00 |  |
| nextheaderoffset | DWORD | 44.00 |  |
| hollerith | char [80] | 48.00 | 0.00 |
| index[0] | DWORD | 128.00 | 0, 68 |
| index[1] | DWORD | 132.00 | 1,68 |
| ... | DWORD | 136.00 | 2,68 |